To Repair your hearing aids with your iPhone

Settings

General (new phone/updates Accessibility is right under settings)

Accessibility

Hearing Devices

(your name) hearing aids

At bottom of screen press FORGET DEVICE

Press FORGET

Go to the previous screen

Place aids in charger, remove after 10 seconds OR

Hold button down on each hearing aid for 10 seconds to turn off

Hold button down on each hearing aid for 10 seconds to turn on

(they are now in pairing mode)

Place aids on top of phone

(your name) hearing aids comes up

Go to next screen (click on not paired)

When Pair comes up touch the word Pair for each hearing aid

Make sure the Signia App is closed out*

Then open the Signia App

Everything should be working now.

If not, call the office:

203-791-2020 x255 or the Signia consumer Bluetooth hotline at 800-350-6093

*to completely close out an app(s) on your iPhone:

double click the button on the bottom of the iPhone or on newer phones swipe up slowly and all open apps should come up.

swipe the app(s) up off the phone

re-open the app